WORKSHOP

RESEARCH

ASSIGNMENT

Flores, Spyke Archel M.

BSME-1B

Prof. Ruel Aggabao

Types of gear

1. Spur Gear

Definition:
A spur gear has straight teeth cut parallel to the axis of rotation. It is the simplest and most commonly used gear.

Use:

- Used to transmit power between parallel shafts

- Found in clocks, gearboxes, conveyors, and simple machinery

Key points:

- Easy to manufacture

- Efficient

- Noisy at high speeds

2. Helical Gear

Definition:
A helical gear has teeth cut at an angle (helix) to the axis, allowing smoother contact between gears.

Use:

- Used in automobile transmissions

- Industrial machinery requiring quiet operation

Key points:

- Quieter than spur gears

- Can carry more load

- Produces axial (side) thrust

3. Double Helical (Herringbone) Gear

Definition:
A gear made of two helical gears joined together with opposite angles.

Use:

- Heavy-duty applications like marine engines and turbines

Key points:

- Eliminates axial thrust

- Smooth and efficient

- Expensive to manufacture

4. Bevel Gear

Definition:
A bevel gear is used to transmit motion between intersecting shafts, usually at 90°.

Use:

- Differentials in vehicles

- Hand drills and right-angle drives

Key points:

- Changes direction of rotation

- Comes in straight, spiral, and zerol bevel types

5. Worm Gear

Definition:
A worm gear consists of a worm (screw) and a worm wheel.

Use:

- Lifts, elevators, conveyor systems

- Speed reduction systems

Key points:

- High speed reduction

- Smooth and quiet

- Low efficiency due to friction

- Often self-locking

6. Rack and Pinion Gear

Definition:
This system consists of a round gear (pinion) and a flat linear gear (rack).

Use:

- Steering systems in vehicles

- CNC machines and sliding gates

Key points:

- Converts rotary motion to linear motion

- Simple and accurate

7. Planetary (Epicyclic) Gear

Definition:
A planetary gear system has a sun gear, planet gears, and a ring gear.

Use:

- Automatic transmissions

- Robotics and aerospace applications

Key points:

- Compact and powerful

- High torque transmission

- Multiple speed ratios

8. Internal Gear

Definition:
An internal gear has teeth cut on the inside surface of a cylinder.

Use:

- Used in planetary gear systems

- Compact machinery

Key points:

- Smooth operation

- Saves space

9. Hypoid Gear

Definition:
A hypoid gear is similar to a bevel gear but with non-intersecting shafts.

Use:

- Rear axle drives in automobiles

Key points:

- Handles high torque

- Smooth and quiet

- Requires special lubrication

10. Spiral Gear

Definition:
Spiral gears have curved teeth and are used for non-parallel, non-intersecting shafts.

Use:

- Light-load power transmission

Key points:

- Quiet operation

- Low efficiency

- Limited load capacity

Spur Gear

Definition:
A spur gear is a type of gear that has straight teeth cut parallel to the axis of rotation. It is the most basic and commonly used gear.

How it works:
When two spur gears mesh, their teeth come into contact all at once, transmitting motion and power from one shaft to another. The shafts must be parallel.

Uses:

- Clocks and watches

- Gearboxes

- Conveyors

- Simple machines and industrial equipment

Advantages:

- Simple design

- Easy and cheap to manufacture

- High efficiency

Disadvantages:

- Noisy at high speeds

- Not suitable for very high load or speed

Nomenclature

Nomenclature means the technical terms used to describe the different parts and dimensions of a gear.

Below are the important spur gear nomenclature terms

1. Pitch Circle

An imaginary circle that rolls without slipping with the pitch circle of another gear.

2. Pitch Diameter

The diameter of the pitch circle.

3. Addendum

The radial distance between the pitch circle and the top of the tooth.

4. Dedendum

The radial distance between the pitch circle and the bottom of the toot

5. Addendum Circle

The circle passing through the top of the gear teeth.

6. Dedendum (Root) Circle

The circle passing through the bottom (root) of the gear teeth.

7. Tooth Thickness

The width of the tooth measured along the pitch circle.

8. Space Width

The distance between two adjacent teeth, measured along the pitch circle.

9. Circular Pitch

The distance along the pitch circle between corresponding points of adjacent teeth.​

where:

- DDD = pitch diameter

- TTT = number of teeth

10. Module

The ratio of pitch diameter to the number of teeth.

11. Base Circle

The circle from which the involute tooth profile is generated.

12. Face Width

The width of the gear tooth measured parallel to the axis.

13. Clearance

The difference between dedendum and addendum of mating gears.

14. Working Depth

The sum of the addendum of the two mating gears

Process of gear cutting

Gear cutting is the manufacturing process of forming gear teeth on a gear blank.

Main Gear Cutting Methods:

- Form Cutting

  - Uses a form-relieved cutter

  - Each cutter is designed for a specific tooth shape

  - Simple but less accurate

- Hobbing

  - Uses a hob (worm-like cutter)

  - Continuous cutting process

  - Most commonly used method

- Shaping

  - Uses a gear-shaped cutter

  - Suitable for internal and external gears

- Milling

  - Done on a milling machine

  - Uses a gear cutter and indexing

Gear Calculation

Gear calculations are used to determine gear dimensions.

Important Gear Formulae:

- Module (m)

m=D/T ​

- Pitch Circle Diameter (D)

D=m×T

- Addendum

=m

- Dedendum

=1.25m

- Circular Pitch (p)

p=πmp

- Tooth Thickness

=P/2​

Gear Cutters

Gear cutters are special cutting tools used to cut gear teeth.

Types of Gear Cutters:

- Involute Gear Cutter

  - Used on milling machines

  - Available in sets (No.1 to No.8)

  - Each cutter cuts a specific range of teeth

- Hob Cutter

  - Used in hobbing

  - Worm-shaped cutter

- Shaper Cutter

  - Used in gear shaping

  - Looks like a gear

Gear Plates

Gear plates (Index plates) are circular plates with holes used in indexing.

Features:

- Made of hardened steel

- Have several concentric circles of holes

- Each circle has a different number of holes

- Used with a dividing head

Purpose:

- To accurately divide the gear blank

- Ensures equal spacing of teeth

Indexing

Indexing is the process of dividing the circumference of a gear blank into equal parts.

Types of Indexing:

- Simple Indexing

  - Most commonly used

  - Uses index plate and dividing head

- Compound Indexing

  - Used when simple indexing is not possible

- Differential Indexing

  - Used for special numbers of teeth

Simple Indexing Formula:

Crank movement=40/T Where:

- 404040 = dividing head ratio

- TTT = number of teeth